Copyright 2019 by The American Institute of Architects (AIA)

Exclusively published and distributed by AVITRU, LLC for the AIA

SECTION 077619 - PORCELAIN-TILE ROOF PAVERS

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - SUMMARY

        - Section Includes:

          - Porcelain-tile roof pavers.

          - Paver support system.

          - Supplementary support system components and accessories necessary for installation.

          - Other system components provided by Installer.

      - ACTION SUBMITTALS

"Action submittals" are submittals requiring responsive action and return of reviewed documents to Contractor.

        - Product Data: For each type of product and system indicated.

          - Include manufacturer's specifications for materials, finishes, construction details, installation instructions, and recommendations for maintenance.

        - Shop Drawings: For paver tile and paver support system layout; indicate adjacent construction, perimeter containment components, penetrations, attachment details, and other conditions.

          - Include data indicating compliance with performance requirements.

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection: For each type of porcelain-tile roof paver, provide representative charts of manufacturer's full range of patterns and colors.

        - Samples for Verification: For each type of exposed finish for each porcelain-tile roof paver selected.

          - Paver Size: Manufacturers' standard size, [4 by 4 inches] [12 by 12 inches] [24 by 24 inches][full].

          - Accessories: [Adjustable pedestal support] [extension rings] [aluminum track] [plate bearings].

Retain "Delegated Design Submittal" paragraph below if design services have been delegated to Contractor. Tile Tech does not provide delegated design services.

      - INFORMATIONAL SUBMITTALS

"Informational submittals" are submittals that do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor. All informational submittals must be reviewed by Architect same as action submittals.

Coordinate "Qualification Statements" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article. If inserting additional entities or specialist, add qualifications to "Quality Assurance" Article.

        - Qualification Statements: For delegated design engineer.

        - Sample Warranty: For warranty review.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data: For porcelain-tile roof pavers and paver support system to include in operation and maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

Verify that Owner wants the extra stock materials, has the storage area for them, and wants to pay for them. If included in Project, verify the percentage amount for the extra materials - 2 percent is standard. Revise as necessary to reflect other amount.

        - Extra Stock Material: Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents. Include inventory list of items delivered.

          - Porcelain-Tile Roof Pavers: Full-size units in quantity equal to [3] <Insert number> percent of quantity installed for each type, pattern, color, and size.

          - Paver Supports: Units in quantity equal to [3] <Insert number> percent of quantity installed for each type and size. Include manufacturer-furnished adjustment tool.

          - Deliver materials to location determined by Owner.

      - QUALITY ASSURANCE

Retain "Delegated Design Engineer" paragraph below to suit Project.

        - Delegated Design Engineer: A professional engineer who is legally qualified to practice in jurisdiction where Project is located and who is experienced in providing engineering services of the kind indicated.

      - MOCKUPS

        - Mockup: Build mockups to verify selections made under Sample submittals and to set quality standards for materials and execution.

          - Build mockup in size and location indicated on Drawings, or if not indicated, no fewer than [four] <Insert number> porcelain-tile roof pavers, [three]<insert number> aluminum tracks,  and [nine] <Insert number> roof paver supports.

          - Acceptance of mockups does not constitute acceptance of deviations from the Contract Documents contained in mockups unless Architect specifically accepts such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

          - Subject to compliance with requirements, accepted mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

      - DELIVERY, STORAGE, AND HANDLING

        - Protect products during shipping, handling, and storage to prevent staining, chipping, deterioration of components, or other damage. Store unused materials in original packaging.

      - WARRANTY

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Manufacturer's Warranty: Manufacturer and Installer agree to repair or replace the following:

          - Damaged porcelain-tile roof pavers that are delivered to Project site chipped, cracked, or broken prior to being unpackaged.

          - Porcelain-tile roof pavers and paver supports that fail in materials or workmanship within specified warranty period when installed and used in accordance with manufacturer's written instructions.

Verify available warranties and warranty periods for porcelain-tile roof paver units and support components.

          - Manufacturer's Warranty Period: Lifetime from date of Substantial Completion.

          - Installer's Warranty Period: [Three] <insert number> years from date of Substantial Completion.

- PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Avitru. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

      - SOURCE LIMITATIONS

        - Obtain porcelain-tile roof pavers and roof paver supports from single source or producer.

      - PORCELAIN-TILE ROOFING PAVERS

Porcelain-tile roof pavers can be installed over all types of roofing membranes. Contact Tile Tech to determine if a protection course is required over the specified roofing membrane type to prevent damage to roofing by paver support system.

        - Structural Porcelain-Tile Roof Pavers: An impervious vitreous tile with a water-absorption rate of 0.5 percent or less in accordance with ASTM C373 test method; abrasion, freeze/thaw, chemical, and stain resistant; intended for exterior use with paver support system.

Tile Tech's "Porcelain Pavers Series" provides exceptional strength, stain resistance, scratch resistance, frost resistance, and low maintenance in many colors and textures; see www.tiletechpavers.com/porcelain-pavers

Tile Tech also manufactures "Ipe Deck Tiles," "Concrete Pavers," and other paver types including "Cool-Roof Pavers," "Turf Trays," and "Plant Trays."

Tile Tech's "Concrete Pavers" include:

"Granite-Tech Pavers," see www.tiletechpavers.com/granitetech-pavers.

"Stamp-Tech Pavers," see www.tiletechpavers.com/stamptech-pavers.

"Oceanic-Series Pavers," see www.tiletechpavers.com/oceanic-series-pavers.

"Recycled-Glass Pavers," see www.tiletechpavers.com/recycled-glass-pavers.

Retain "Basis-of-Design Product" subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

          - Basis-of-Design Product: Subject to compliance with requirements, provide MBRICO LLC Porcelain Pavers with FRP substrate plate [Stone] [Silverlake] [Esprit] [Soul] [Glocal] [Quartzite] [Mood Wood] [Verve] [Quartz] [Essence] [Quartz Design] [Slate] [Bluestone Design] [Denverstone] [Quartziti 2.0] [Milano City] [Mashup] [+One] [Nau] [Travertine] [Flagstone] [Earth] [Stones 2.0] [Officine] [Concrete] [Marble] [Aura] [Wood] Series or comparable product by one of the following:

            - <Insert manufacturer's name>.

          - Size: [23.6 by 23.6 inches] [23.6 by 47.2 inches] [15.75 by 47.2 inches].

          - Thickness: 1 inch

          - Weight: 10.5 lbs. /sq. ft.

Porcelain-tile pavers may shatter and collapse due to sudden impact. Contact Tile Tech for safety measures in the event of tile breakage, such as using Tile Tech's "Hex-Tray Wind Uplift and Safety System" below tiles for installations using support pedestal systems of 6 inches (150 mm) in height or greater. Tile Tech's "Hex-Tray" system has been tested to prevent complete failure and collapse of the tiles to the top of roofing when shattered by impact. Safety system consists of honeycomb structural trays combined with Tile Tech's "Hybrid" adjustable-height paver pedestals, with adhesive in accordance with manufacturer's written instructions used to adhere porcelain pavers to trays. For additional information, see www.tiletechpavers.com/Hex-Tray_WindUplift.pdf.

For Tile Tech's "Hex-Tray" test results, see www.tiletechpavers.com/Porcelain-Paver-Test-Load_C293.pdf.

          - Breaking Strength (ISO 10545-4): 4500 lb./sq. ft. minimum.

          - Static Load Capacity (EN 12825): 1700 lb./sq. ft. minimum.

          - Slip Resistant (ASTM C1028): R11 to R13 rated

          - Wind Uplift: (ASTM E 1592): 330 psf

For Tile Tech's tile selection options, see www.tiletechpavers.com/Porcelain-Paver-Catalog.pdf.

          - Colors and Textures: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's standard range] [As selected by Architect from manufacturer's custom range] [As selected by Architect from manufacturer's full standard and custom range] <Insert color>.

      - PAVER SUPPORT SYSTEM

For additional paver support information, see www.tiletechpavers.com/adjustable-pedestals-hybrid and www.tiletechpavers.com/deck-pedestals-standard.

For Tech Tile's "Hybrid Pedestal" support system details, see www.tiletechpavers.com/Hybrid-Pedestal-Details.pdf.

        - Adjustable-Height-Pedestal Paver Supports: Continuously variable, adjustable-height, high-impact and flame-resistant ABS plastic system components. Accommodate height adjustments from 1/2 to 39 inches to provide level paver installation. System shall maintain 1/8- to 1/4-inch paver joint width and alignment of pavers while allowing for drainage, air circulation, and preventing water accumulation.

Retain "Basis-of-Design Product" subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

          - Basis-of-Design Product: Subject to compliance with requirements, provide MBRICO LLC NM Adjustable Pedestals or comparable product by one of the following:

            - <Insert manufacturer's name>.

          - Load Capacity: Not less than 3300 lbs .

        - Paver Support System Components and Accessories: Provide manufacturer's standard high-impact and flame-resistant ABS plastic spacer tabs, shims, extenders, aluminum track and lateral bracing support members, and other accessories as necessary for a complete assembly.

In high-wind areas, consider using Tile Tech's "Hex-Tray Wind Uplift and Safety System" as described in "Porcelain-Tile Roofing Pavers" Article above. Consult Tile Tech on whether its "Hex-Tray" should be incorporated for installations using support pedestal system of 6 inches (150 mm) in height or greater. Consult Tile Tech for use of lateral support bracing system for pedestal support systems of 24 inches (610 mm) in height or greater.

          - Extension Rings:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

            - Basis-of-Design Product: Subject to compliance with requirements, provide MBRICO LLC Extension Rings or comparable product by one of the following:

              - <Insert manufacturer's name>.

          - Plate Bearings:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

            - Basis-of-Design Product: Subject to compliance with requirements, provide MBRICO LLC Plate Bearings or comparable product by one of the following:

              - <Insert manufacturer's name>.

          - Aluminum Track System:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

            - Basis-of-Design Product: Subject to compliance with requirements, provide MBRICO LLC Aluminum Track System, T Tracks, L Tracks, and Offset L Tracks, or comparable product by one of the following:

              - <Insert manufacturer's name>.

      - Other System Components, Provided by Installer

        - Protection Course: As recommended by roofing membrane manufacturer to prevent damage to roofing while allowing adequate flow to drainage system.

        - Perimeter Containment Components: Typical, but not required: System as provided by Installer to restrain paver support system.

      - PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

        - Delegated Design: Engage manufacturer (MBRICO LLC) to design paver support system, including pedestal placement and track layout.

Retain "Paver Support System" Paragraph below, along with "Delegated Design" Paragraph above, when Contractor is required to take responsibility for structural design of installation. Revise to correspond to requirements of authorities having jurisdiction.

        - Paver Support System: Capable of withstanding effects of gravity loads and the following loads and stresses, within limits and under conditions indicated:

          - Floors: Uniform load of 125 lbf/sq. ft. (610 kg/sq. m) or a concentrated load of 2000 lbf (8.9 kN), whichever produces greater stress.

          - Walkways and Elevated Platforms Other Than Exits: Uniform load of 60 lbf/sq. ft. (293 kg/sq. m).

          - Walkways and Elevated Platforms Used as Exits: Uniform load of 100 lbf/sq. ft. (488 kg/sq. m).

Retain "All Roof Zones (Corner, Perimeter, and Field-of-Roof) Uplift Pressures" Subparagraph below and delete "Roof Zone 1 (Field-of-Roof) Uplift Pressure," "Roof Zone 2 (Perimeter) Uplift Pressure," and "Roof Zone 3 (Corners) Uplift Pressure" subparagraphs, if roof wind uplift pressures are indicated on Drawings.

For wind uplift information on Tile Tech porcelain pavers, see www.tiletechpavers.com/Hex-Tray_Wind%20Uplift.pdf.

          - All Roof Zones (Corner, Perimeter, and Field-of-Roof) Uplift Pressures: As indicated on Drawings.

          - Roof Zone 1 (Field-of-Roof) Uplift Pressure: <Insert lbf /sq. ft. (kg/sq. m)>.

          - Roof Zone 2 (Perimeter) Uplift Pressure: <Insert lbf /sq. ft. (kg/sq. m)>, located within <Insert ft. (m)> of roof perimeter.

          - Roof Zone 3 (Corners) Uplift Pressure: <Insert lbf /sq. ft. (kg/sq. m)>, located within <Insert ft. (m)> of roof outside corner.

Retain "All Roof Zones (Corner, Perimeter, and Field-of-Roof) Uplift Pressures" Subparagraph below and delete "Roof Zone 1 (Field-of-Roof) Uplift Pressure," "Roof Zone 2 (Perimeter) Uplift Pressure," and "Roof Zone 3 (Corners) Uplift Pressure" subparagraphs, if roof wind uplift pressures are indicated on Drawings.

For wind uplift information on Tile Tech porcelain pavers, see www.tiletechpavers.com/Hex-Tray_Wind%20Uplift.pdf.

        - Perimeter Containment System: Capable of resisting lateral forces in paver support system without displacement exceeding 1/8 inch.

      - MATERIALS

        - ABS Plastic: Complying with ASTM F2806.

- EXECUTION

      - EXAMINATION

        - Examine substrates and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

          - Verify suitability of any perimeter containment components.

        - Examine system installation, with Installer present, for protection from paver support system installation. Examine areas where roofing system is turned up or flashed against vertical surfaces and horizontal roofing.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Coordinate placement of paver support system with roofing membrane Work.

        - Lay out porcelain-tile roof paver and roof-paver support locations, and mark for installation.

      - INSTALLATION OF PAVER SUPPORT SYSTEM

        - Install protection course and / or bearing pads below pedestals in accordance with manufacturer's written instructions to protect roofing substrate surface.

        - Accurately set paver supports in locations coordinated with approved porcelain-tile roof paver layout. Install track system in accordance with manufacturer's written instructions and accepted Shop Drawings. Adjust paver support heights prior to, and following, installation of porcelain-tile roof pavers. Shim where fine adjustment is necessary using manufacturer-provided shims.

Retain paragraph below for installations in high-wind areas, for "safety" installations using pedestal support system of 6 inches (150 mm) in height or greater, and for pedestal support systems of 24 inches (610 mm) in height or greater. Revise to suit Project.

        - Install lateral bracing supports in accordance with manufacturer's written instructions and accepted Shop Drawings.

      - INSTALLATION OF ROOF PAVERS

        - Install porcelain-tile roof pavers over support system in accordance with manufacturer's written instructions and accepted Shop Drawings. Align joint patterns parallel in each direction. Lay out paver units to avoid less than half-width units at perimeter or other terminations.

        - Set porcelain-tile roof pavers in place using placement methods that result in stable installation free from rocking, using sound pavers with no edge or surface damage. Make final in-place level adjustments using manufacturer's furnished tool to adjust paver support heights.

        - Do not use paver units with chips, cracks, voids, discolorations, and other defects that might be visible or cause staining in finished work.

      - CONSTRUCTION TOLERANCES

        - Variation of Level Across Width of Each Porcelain-Tile Roof Paver: 1/16 inch maximum.

        - Variation from Level for Paved Areas: Do not exceed 1/4 inch in 10 ft. when measured at any location and in any direction.

        - Variation in Joint Width: Do not vary joint thickness more than one-fourth of nominal joint width.

        - Variation in Plane at Joints (Lipping): Do not exceed 1/16-inch difference between planes of adjacent units.

        - Variation in Line of Edge at Joints (Lipping): Do not exceed 1/16-inch difference between edges of adjacent units, where edge line continues across joint.

      - CLEANING AND PROTECTION

        - Remove and replace loose, chipped, broken, stained, or otherwise damaged porcelain-tile roof pavers or pavers that do not match adjoining units or pattern indicated on Drawings.

        - Cleaning: Remove soiling from exposed porcelain-tile roof paver surfaces; wash and scrub clean. Leave joints between pavers open and clean of debris to allow for proper drainage and airflow.

        - Provide final protection and maintain porcelain-tile roof pavers without damage or deterioration at time of Substantial Completion.

END OF SECTION 077619